# DETAILED DESIGN DOCUMENT

Distributed Multi-Robot Tracking Handoff System

Sections 1 & 2 — Data & Communication Protocol / Negotiation & Decision Logic

Detailed Design Phase — R&D Blueprint

Figure 1. High-level decentralized handoff architecture.

# SECTION 1 — Data & Communication Protocol

## 1.1 MQTT Topic Taxonomy

A scalable topic space separates traffic by function first, then entity, then stream. This allows clean subscriptions and class-specific QoS policies.

Figure 2. Proposed three-tier MQTT topic hierarchy.

### Proposed Topic Structure

fleet/ ├── discovery/ │ └── heartbeat ├── <robot_id>/ │ ├── telemetry/ │ │ ├── pose │ │ ├── battery │ │ └── status │ └── vision/ │ └── tracks/<track_id> └── handoff/ └── <track_id>/ ├── announce ├── bid ├── award └── ack

Key design choice: negotiation topics are keyed by track_id, enabling a robot to subscribe only to relevant target negotiations instead of fleet-wide chatter.

### Topic Classes & QoS

| Topic class | QoS | Retained | Purpose |
| --- | --- | --- | --- |
| telemetry/pose, telemetry/battery | 0 | No | High-frequency latest-state data |
| vision/tracks/* | 1 | No | Track metadata; duplicates tolerated |
| handoff/*/announce, bid | 1 | No | Negotiation messages |
| handoff/*/award | 2 | Yes (short TTL) | Critical winner declaration |
| discovery/heartbeat | 0 | Yes | Latest fleet liveness snapshot |

## 1.2 Lean JSON Payload Schema

The dominant payload cost is the 128-dimensional Re-ID embedding. Quantizing the vector to INT8 reduces its raw size to 128 bytes, while short keys reduce repeated field-name overhead.

Figure 3. Lean payload composition and serialization path.

### Proposed Bid Payload

{ "tid": 17, "rid": "R04", "ts": 1893456213, "seq": 42, "pos": [12.4, 5.1], "vel": [0.8, -0.2], "conf": 0.91, "batt": 0.63, "los": 0.78, "u": 0.712, "reid": "b64:qk3F...==" }

### Re-ID Quantization

q_i = round(v_i × 127), q_i ∈ [-127, 127] v_i ≈ q_i / 127

Size comparison: float64 ≈ 1024 bytes; float32 ≈ 512 bytes; INT8 = 128 bytes for the 128-dimensional vector.

## 1.3 Serialization & Compression Strategy

| Option | Approx. payload | Decision |
| --- | --- | --- |
| JSON, float64 + verbose keys | ~2,600 B | Baseline — rejected |
| JSON, INT8 Re-ID via base64 | ~330 B | Fallback |
| CBOR, native byte-string | ~155 B | Recommended |
| Protobuf | ~150 B | Future option after schema freeze |
| MessagePack | ~150 B | Comparable; CBOR preferred |

Recommendation: use CBOR for latency-critical handoff and track payloads during the simulation phase, with a possible migration to Protobuf once the message contract becomes stable.

General-purpose compression is not recommended for the INT8 Re-ID vector itself because the quantized embedding is effectively noise-like for deflate-style compressors; compression remains useful for batched telemetry.

# SECTION 2 — Negotiation & Decision Logic (NegotiationSERVICE)

Figure 4. Decentralized negotiation and winner-selection flow.

## 2.1 Utility Score Cost Function

The utility score is a single scalar in [0,1] that ranks candidate robots using normalized distance, line-of-sight probability, battery condition, and current tracking load.

U_i = w1 · f_dist(d_i) + w2 · f_los(p_i) + w3 · f_batt(b_i) + w4 · f_load(n_i) subject to: w1 + w2 + w3 + w4 = 1, U_i ∈ [0, 1]

### Component Functions

d_i = ¦¦ p_robot_i − p_target(t + Δt_negotiation) ¦¦₂ f_dist(d_i) = max(0, 1 − d_i / d_max) f_los(p_i) = p_i, p_i ∈ [0,1] f_batt(b_i) = b_i^γ, γ > 1 (e.g., γ = 2) f_load(n_i) = 1 − (n_i / n_max)

The distance is computed to the target's predicted position at handoff time rather than its current position, accounting for target motion during negotiation latency.

### Proposed Initial Weights

| Term | Weight | Rationale |
| --- | --- | --- |
| Distance (w1) | 0.35 | Strong predictor of reacquisition and sustained tracking |
| LOS (w2) | 0.30 | Core objective of proactive handoff |
| Battery (w3) | 0.20 | Operational reliability; shaped non-linearly |
| Load (w4) | 0.15 | Prevents overloading already-busy robots |

These values are engineering starting points, not experimentally proven optima. They should be exposed as runtime parameters and tuned using simulation-based sensitivity analysis.

## 2.2 Decentralized Tie-Breaking Mechanism

Because utility values are quantized and near-equal scores are possible, the system uses an epsilon-band followed by a deterministic hash-based secondary key.

1) If ¦U_i − U_j¦ ≤ ε, treat the scores as tied (e.g., ε = 0.01). 2) key_i = hash(track_id ⊕ robot_id ⊕ seq) 3) winner = argmin_i key_i

Hash-based tie-breaking avoids systematically favoring the same low-numbered robot and distributes tie wins across the fleet.

### Convergence Rule

Candidates subscribe to handoff/<track_id>/bid for a fixed T_negotiate window.

Each candidate maintains the best (utility, key) tuple observed, including its own bid.

If its own bid remains best when the window closes, it publishes the award.

Other candidates stop bidding and the winner publishes ACK.

If no award is observed, candidates re-derive the winner locally and re-announce as a self-healing safety net.

## Design Validation Priorities

| Area | Metrics / experiments |
| --- | --- |
| Communication | Payload size, serialization latency, MQTT end-to-end latency, packet loss |
| Negotiation | Decision latency, winner consistency, message count, failure recovery |
| Prediction integration | TTO error, occlusion detection accuracy, trigger latency |
| End-to-end handoff | Success rate, target ID consistency, tracking interruption duration |

## Next Steps

Validate the schema against the ROS 2 ↔ MQTT bridge selected for the Gazebo testbed.

Run sensitivity sweeps for w1…w4, T_negotiate, T_jitter, and the epsilon tie threshold.

Benchmark JSON versus CBOR under realistic wireless payload and latency conditions.

Integrate the negotiation flow with the trajectory/occlusion PredictionSERVICE and Level-1 Gazebo scenarios.